*****************************************************************************

Axe-Fx Firmware Release Notes

>>>>> To encourage people to actually read these, firmware versions previous to >>>>> 5.00 are now archived.

*****************************************************************************

5.24

Fixed some knob positions not set properly when modified in Drive, Format, Synth and Quad-Chorus blocks.

*****************************************************************************

5.23

NOTE: Support for presets created with Version 3.xx or earlier firmware has been dropped.  If you are upgrading from Version 3.xx or earlier, upgrade to 5.22 first and run the “Update All Presets” utility in the Utility->Preset menu.

Changed range of frequencies on Resonator block to allow more varied tones.  Audition any existing presets that use the Resonator block.

Added Mono|Stereo input select to Resonator block.  In Mono mode (default) left and right are summed and fed to all four resonators.  In stereo mode the left intput is fed to the first two resonators and the right is fed to the last two.

Improved preset change behavior to reduce pops and clicks.

*****************************************************************************

5.22

Changed Delay block algorithm to eliminate zippering on tempo changes.  Reworked modulation for better sound quality.

*****************************************************************************

5.21

Fixed value strings being overwritten by Delay block when modifier attached to certain parameters.  This typically showed up in the Global Amp type being overwritten with a percent value.

Improved Reverse Delay algorithm.  Doubled the available snippet length, changed crossfade method and improved overall performance and sound quality.  Phase Reverse parameter now works as well.  NOTE: Crossfade range has been changed from 1 to 255 ms.  Audition existing presets.

Added modifiers to Dual Delay Left and Right level controls.

Added Mix control to compressor.  For presets created prior to this release the mix value will be zero.  You will need to adjust the mix to the desired amount and re-save the preset.  Set to 100% for pre-5.21 sound.

Added Mix control to Drive block.

Added new tonestack placement option “END”.  This places the tonestack after the power amp simulation.

Added Warmth parameter to Power Amp simulation.  This parameter can be found in the Advanced menu and controls the power tube and transformer interaction.  Set to zero for pre-5.21 sound.  Increasing this value increases the apparent warmth and depth of the tone, especially as the power amp is driven harder.  This parameter defaults to a non-zero value whenever a new amp type is chosen.

Fixed bug in Whammy where pitch would be stuck.  Only affected standard models.

*****************************************************************************

5.20 (internal release)

Improved Reverb algorithm by adding initial density control.

*****************************************************************************

5.19

Fixed vector being clobbered bug in Out1-to-Out2 copy feature.

*****************************************************************************

5.18

Fixed Global Mix parameter not working in Ringmod and Resonator blocks.

Fixed level of Output2 6 dB greater than Output1 when using Out1-to-Out2 copy and Out1 set to L+R sum.

Improved power amp modeling by improving speaker interaction model.

*****************************************************************************

5.17

Fixed distortion in delay block if a very low value of modulation depth is being used.

Fixed vector corruption in Out1-to-Out2 copy feature when using certain routings.

*****************************************************************************

5.16

Added diffuser to Quad-Tap algorithm.  The diffusor is located at the input of the delay line.  This can be used to smear the echoes.  Diffusion sets the mix level and Diff Time sets the length of the diffusers.

Added code to defeat filtering in Delay block if Lowcut and Highcut are at their respective miminum and maximum values.  This allows for infinite delays with no degradation.

Added ability to attach modifier to Drive block Drive control.

Added “Copy Out1 to Out2” feature.  This parameter is located in the I/O->Audio menu.  When set to ON the data at the main outputs (Output1) is copied to Output2 if the effects loop is not in use.  The data at Output2 is affected by the Output2 Global EQ but not the Output1 Global EQ.  This allows easily configuring a separate stage monitor and FOH send.  Use Output1 to FOH and Output2 to stage monitor.  If the effects loop is in use this parameter is ignored.

Fixed Mega-Tap time and level values being corrupted on edit/recall.

Fixed unequal L/R levels when Output1 mode set to SUM L+R.

*****************************************************************************

5.15

Reprocessed all cabinet impulse responses to increase dynamic range.  Reworked cabinet block to take advantage of increase.

Added “4x12 CALI” cabinet model.  Nice sample of a Mesa Traditional cab.

*****************************************************************************

5.14

Fixed verify on preset backup to flash incorrectly stating failed (standard only).

*****************************************************************************

5.13

Fixed Ducker causing exception when Threshold at 0.0 (divide-by-zero).

Fixed Global Levels conflicting with MIDI map at 12 and 21.  NOTE: you may need to reset your levels after update.

*****************************************************************************

5.12

Fixed Bright, Deep, etc. “pull” switches not being saved when using Global amp.

Fixed Quad Chorus crashing if all rate multipliers at minimum and master rate very low.

*****************************************************************************

5.11

Fixed 1x12 Thiele IR accidentally compiled into 1x6 Oval cabinet slot.

Fixed mild zipper noise in panner when using extreme widths/rates.

Added verfiy step to flash process.  Added retry if verification fails.  Unit now automatically reboots when flash is complete.

Added slightly more range to LCD contrast control.

*****************************************************************************

5.10

Fixed Dual Delay pan values not “sticking”.

Added ducking capability to Multidelay block.  Existing presets using the Multidelay will need to be auditioned.  Turning “Ducker Atten” to 0.0 dB will defeat the ducker.  Most, if not all, exisiting presets should have the attenuation set to 0 dB but you should check to be sure.

Added Gain control to Global Output EQ’s.  This can be used to reduce/increase the overall level of all presets.  When boosting bands on the Global EQ’s, output clipping can occur.  This control allows you to compensate for this.  The value of this parameter should be verified after installing the new firmware.

Fixed crash when using “Vintage” tone-stack due to arithmetic exception.

*****************************************************************************

5.09

Improved power amp modeling by more accurately modeling high-frequency interaction with speaker impedance.

*****************************************************************************

5.08

Fixed Volume blocks taper not sticking.

*****************************************************************************

5.07

Fixed CC bypass of effects not working properly if autoengage on and Ignore Redundant Program Change true.

Added MIDI bypass of Volume blocks.  Volume1-4 now show up in I/O->MIDI menu and will likely need to be set/initialized.  Wah1,2 controller numbers will also likely need to be reset.

Reduced autoengage point to 15%.

*****************************************************************************

5.06

Fixed loading single preset via MIDI hang.

Returned Autoengage to pre-5.05 operation and changed engage point to 20%.

Added Marshall Bluesbreaker and Shredmaster Drive models.

*****************************************************************************

5.05

Fixed Multiband Compressor mute bypass mode not muting.

Mute output if exceeding CPU limit to prevent “stuck vector” squeal.

Added Master Pan parameter in Dual mode for Delay block.

Changed bypass control so that setting bypass state via MIDI operates the same as setting state via front panel.  This will cause Edited LED to light and is considered normal operation.

Vectorized wet/dry values to eliminate chirping and volume surges when engaging/bypassing effects.

Totally reworked Drive block.  Changed tone control to passive emulation.  Updated/tweaked all models.  Fixed mistakes in Big Muff Pi and Boss SD-1 models.  Changed names of some models.  Removed old Super OD model and replaced with new Boss SD-1 model.  Put MXR Distoriton+ where old SD-1 model was.

Changed Drive block level control position to eliminate volume surges when engaging/bypassing.

Fixed clicking/popping on preset changes to presets using Multidelay block due to shared semaphore.

Fixed delay spilling over when delay spill off if delay time very long.

Changed Global EQ’s to 8-band Graphic.  Moved phase controls to I/O menu. NOTE: If updating firmware the 8-band Graphic EQ’s will need to be set/updated (double-click BYPASS to zero all bands).

Fixed Autoengage incorrectly engaging blocks due to “leftover” data from previous preset.

Reworked Autoengage to operate better.  Now engages at any point above 5%.

Fixed Pitch Shifter not shifting correctly for certain scales due to incorrect min/max shift limits.  Fixed custom scales not loading right on preset recall due to scale being shared by both blocks and therefore being overwritten.

Fixed Harmonic Minor and Melodic Minor scale type labels swapped.

Added offset nulling to Amp and Drive blocks to reduce clicks and pops on preset changes.

Fixed Reset System Parameters not working due to SRAM lockout.

*****************************************************************************

5.04

Added code to ignore MIDI Active Sense messages.

*****************************************************************************

5.03

Fixed modifiers not being disconnected properly on patch recall.

Added input select to Amp block.

Decreased preset recall latency.

*****************************************************************************

5.02

Fixed shunt not being placed when using editor.

Changed LCD update process to prevent command corruption.

*****************************************************************************

5.01

Added modifier checking to delete bogus modifiers connected to illegal effects/parameters.

Changed preset recall routine to run faster and switch quieter.

*****************************************************************************

5.00

Added passive tone control option to amp blocks.  Global parameter allows choosing between active and passive as default when selecting amp type.

“Reference Class” pitch shifting including:

Improved intelligent pitch shifter, especially for large shifts.

Improved forward/reverse shifter used in Plex Detune, Plex Shift and Crystals algorithms.

Selectable in-block pitch detector allows for detecting pitch at block input giving far greater stability when using pitch feedback.

New hybrid time-domain/frequency-domain pitch detector in pitch block can track chords (Ultra only).

Pitch-tracking now compensates for shift amount resulting in more natural shifts.

Wider filter on pitched notes gives more bell-like and musical quality.

Adjustable low-pass filter.

Fixed shared memory error between pitch blocks.

Master feedback in pitch block now affects crystals algorithm as well.  Existing presets may default to 0%.  Set to 100% for proper operation.

Added mono options to I/O.  User can now select between stereo (default), L+R sum or copy left to right options.

Added many new possible tempos, i.e. 15/16, 1/64., etc.

Changed formant Q range from 2-20 to 4-40.  This allows for a more intense effect.  Existing presets will need to be auditioned due to range change.

Added filter type selection to outer bands on parametric EQ block.  Choices are now shelving, peaking and blocking (i.e. highpass).

Fixed “WRECKER 1” model tone frequency.  Originally it was incorrectly set at 200 Hz.

Fixed Powerball model.  Schematic was intentionally incorrect leading to a myriad of improper values.  Changed name to “ENERGYBALL”.

Added “PLEXI 2” amp model.  Similar to original Plexi model but with different treble peaker on volume control and other minor changes.

Improved biquad and 1st-order filter performances.  Results in smaller footprints and improved execution times.

Improved interpolation algorithm (Ultra only) used in chorus, flanger, and pitch blocks.

Improved allpass filter performance, gives slight improvement to phaser execution speed.

Reworked GUI code to improve effeciency and responsiveness, especially at high CPU loads.

Added RUN parameter to sequencer.  This starts/stops sequencer.  Parameter is modifiable.  NOTE: parameter defaults to OFF and any old presets will need to be updated.

Added tempo controls to the Delay block LFOs.

Added “FET BOOST” type to Drive block.

Added “TAPE DIST” type to Drive block.  This simulates distorion as would be encountered when overdriving a studio reel-to-reel tape recorder.

Reworked Drive block for better sound.

Added delay time parameters to Pitch block for Detune mode.

Added mod ability to dual delay times.